60– 265  

Lab 9 Solution

Q1. A digital computer has a common bus system for 16 registers of 32 bits each.
 The bus is constructed with multiplexers.(a) How many selection inputs are there in   

 each multiplexer? (b) What size of multiplexers are needed? (c) How many multiplexers are there in the bus?

(a) Number of Selection inputs = 4

(b) 16-to-1 multiplexers

(c) 32 multiplexers( one for each bit)

Q2. Explain the memory operation in each of the following register transfer statements

(a) R2 ( M[AR]

(b) M[AR] ( R3

(c) R5 ( M[R5]

(a) Read into the register R2 the memory word at the address in  AR.

(b) Write the contents of register R3, into the memory word specified by the address in AR
(c) Read into the register R5, the memory word specified by the address in R5.

(The process destroys the previous value in R5).

Q3. Design an arithmetic circuit with one selection variable S and two 4 bit data inputs A and B. The circuit generates the following four arithmetic operations, in conjunction with input carry Cin.

                         S                Cin= 0                                     Cin= 1

                         0               D = A + B(add)                      D = A + 1(increment)

                         1               D = A – 1(decrement)           D = A + B’ + 1(subtraction)

(a) Draw  the logic diagram for the first stage. You can use a Full Adder (block diagram) in your design.

[image: image1] 

     S          Cin              X           Y          D


     0            0                A           B          A+B

     0            1                A           0           A+1

     1            0                A           1           A-1

     1            1                A           B’         A-B


[image: image3.png]



(b) Draw a block diagram showing how two such circuits can be connected to form an 8 bit arithmetic unit.

[image: image2]
0  S0     S1


1


   4 to 1MUX


2





3





X1   Cin


      1 bit 


        FA    D1


Y1





        Cout





0  S0       S1


1


   4 to 1MUX





2


3





Xo    Cin


      1 bit 


       FA     D0


Yo  


    


       Cout  





A0





A1





0


1





0


1





Cin





    S





    B0





    B1





output





        S0       S1     Cin                          D0


                                                            D1


                                                             


                                                            D2


                           Cout                         D3





S0      S1            Cin                            D0


                                                            D1


                                                            


                                                           D2


                             Cout                      D3


 





A4 – A7








B4 – B7





       A0 – A7








       B0 – B3





D0


D1





D2


D3





D4


D5





D6


D7





C8








Cin





S








