(0360-265) Computer Architecture
QUIZ – 5     Winter 2008      (Time: 10 mins)

53-C

	Student’s Name:



	Student Number:




[image: image1.png]! 2to1l @ Register .

Register ‘ LA € !

ClnputSel





Notes: All the inputs and outputs of the registers, and the multiplexer (MUX) are 4 bits.
All inputs and outputs are active high.

Q: Are the following register transfer operations feasible? If the operation is feasible, obtain the Boolean expressions (in terms of BusSel0, BusSel1, LoadA, LoadB, LoadC, and CInputSel) for the respective condition.
Condition 1:   
C( A, C( B

Condition 2:   
B( C, C( B 

Condition 3:
A( C, C( A 

Hint: 1. At least two conditions are feasible.

2.  For the 2-to-4 Decoder:

    A = 1, only when BusSel0 = 0 and BusSel1 = 0; 

    B = 1, only when BusSel0 = 1 and BusSel1 = 0;

    C = 1, only when BusSel0 = 0 and BusSel1 = 1.

3. For the 2-to-1 MUX:

     Output = X, when CInputSel = 0;

     Output = Y, when CInputSel = 1









































































