USE THE OTHER SIDE OF THE SHEET, IF NECESSARY.

 (0360-265) Computer Architeture I: Digital Design

QUIZ – 3    IS 2009      (Time: 10 mins)


	Student’s Name:



	Student Number:




Q1(a). A 4-variable Boolean function is given in terms of “ORing of minterms” as follows:

F1(A, B, C, D) = ∑ m(1, 4, 7, 8, 9, 12).

(i) Express the function in terms of the “ANDing of Maxterms”:

F1(A, B, C, D) = π M(0, 2, 3, 5, 6, 10, 11, 13, 14, 15).
(ii) If F1(A, B, C, D) = (F3(A, B, C, D) )’, 

           express the function F3(A, B, C, D) in terms of “ORing of minterms”:

F3(A, B, C, D) = ∑ m (0, 2, 3, 5, 6, 10, 11, 13, 14, 15).
Q1(b). Another 4-variable Boolean function is also given in terms of “ORing of minterms” as follows:

F2(A, B, C, D) = ∑ m(3, 4, 5, 8, 12,).

Express the function 

                           F4 = F1 + F2 

in terms of “ORing of minterms” as follows:

F4(A, B, C, D) = ∑ m(1, 3, 4, 5, 7, 8, 9, 12).

Q1(c). Obtain the Karnaugh map  for F4(A, B, C, D).

	
	CD
	
	
	
	

	AB
	
	00
	01
	11
	10

	
	00
	0
	1
	1
	0

	
	01
	1
	1
	1
	0

	
	11
	1
	0
	0
	0

	
	10
	1
	1
	0
	0


Q 1(d) List the Essential Prime Implicants and Prime Implicants for  

                                                                                               F4(A, B, C, D).

                Prime Implicants: A'D, B'C'D, A'BC', BC'D', AC'D', AB'C'

Essential Prime Implicants: A'D
