CH.5 ADDRESS RESOLUTION PROTOCOL (ARP)

ADDRESS RESOLUTION: The problem of mapping IP addresses to physical (or hardware) addresses.


  


                                                                                    DATA
               IP addresses

                                                                                     TCP/UDP packet

                                                                                     IP DATAGRAM

                                                                                     FRAME  

Physical addresses 

       IP  Address                                                                Physical Address

                                                                                         (Example: Ethernet: 48 bit address)

FIG: Example: FTP Client connected to FTP server

               Sender/Receiver    
             Receiver/Sender









 ETHERNET        NETWORK





ROUTER

FIG: Example: A small part of Internet

                                                                     131.108.22.177
     131.108.154.88



                                    N1: 131.108.0.0

                131.108.0.1

      


                  78.0.0.1

                           N2: 78.0.0.0






                   78.0.0.3                                                        78.0.0.2

                                                                                            




                                                                                                            223.240.129.1

                                                  N4: 223.240.129.0



ADDRESS RESOLUTION
1. Using Table Lookup (STATIC MAPPING) 

Example table:

    

	IP Address
	Hardware Address

	191.21.31.1
	0A:07:4B:12:82:36

	191.21.31.2
	0A:07:4B:12:82:8D

	191.21.31.3
	0A:07:4B:12:82:99

	191.21.31.4
	0A:07:4B:12:82:1F

	191.21.31.5
	0A:07:4B:12:82:28



  
  A separate address binding table for each physical n/w.

	Search Method:
	(i ) Sequential search for a network less the a dozen hosts

	 
	(ii) Hashing

	 
	(iii) Direct indexing used in WAN technologies 


PROBLMES: 1) Changes in NIC

                      2) Mobile Computer

                      3) In some LAN's like local Talk, the PHY ADD changes, every time the computer is turned on.

2. Closed Form Computation 

IA = f (HA)
Used for technologies with configurable address. Example: PRONET
3. Dynamic Mapping 
· ARP 

Given: Target’s IP address.

The target and the host (which wants to use ARP to find the target's physical address) are on the same physical network.

To Find: Target’s physical address.
    

ARP MESSAGE FORMAT
VARIABLE LENGTH FIELDS (28 Octets for Ethernet)   
  

	1
	8
	16
	24
	31

	
  HARDWARE TYPE
	PROTOCOL TYPE

	HLEN
	PLEN
	OPERATION

	SENDER HA (Octets 0 – 3 )

	SENDER HA (Octets 4 – 5)
	SENDER IP (Octets 0 – 1)

	SENDER IP (Octets 2 – 3)
	TARGET HA (Octets 0 – 1)

	TARGET HA (Octets 2 – 5 )

	TARGET IP (Octets 0 – 3 )

	
	
	



  
 Hardware Type: 1 for ETHERNET 

Protocol Type: 080016 for IP 

HLEN, PLEN: Lengths of Hardware & Protocol Address in OCTETS 
  

Operation:

Type of operation   Value of the field

ARP Request                      1

ARP Response                    2

RARP Request                    3

RARP Response                  4

         

	Target HA:
	In ARP Request, it is filled with all Zeros.

	 
	In ARP Response, it contains the HA


Fig:  Ethernet frame for ARP packet:

Ethernet Type for ARP 080616                                      







   









    

PADD-

   











     ING

                                                 

   6            6         2      2       2       1    1      2           6               4             6            4           18       4

                                                                        ARP  Message

ARP Process

RECEIPT by a system of an ARP Request – addressed to the system

· updates the cache with the IP and HA of the Sender

· generates the ARP response

ARP Request is broadcast.

All the machines on the LAN update their ARP cache with the IP & HA of the Sender.

Another use of ARP

USE: A Back-up file server can take over from a failed server by sending an ARP Request with the failed server’s IP Add and the back-up server’s HA.

        ( Bhide, Elnozahy and Morg – 1991)

To create an ARP request

(I) Change OPERATION field value to 2 

(II) Exchange the Sender and Target IP addresses

(III) Put in the HA of the respondent-system

(IV) Put in the HA of the Target, the HA of the machine, which sent the ARP Request

        ( This HA available in the ARP Request in the HA field of the Sender.)        

ARP  PROCESS:

Query (also called the Request) broadcast over the n/w.

Response unicast  to the inquirer.

Example: Using Ethernet to carry ARP Request
ARP REQUEST:

                         D.A.          S. A.        TYPE

	8
	6
	6
	2
	ARP REQ PACKET
	4


    Preamble                                                                                             CRC

DEST- FF:FF:FF:FF:FF:FF

TYPE 0806

ARP Response/Request

4 CASES:

	SENDER
	RECEIVER
	REMARKS

	HOST (A)
	HOST(B1) ON THE SAME NET (N1) 
	

	HOST (A)
	HOST(B2)ON A DIFFERENT NET(N2)
	IP of ROUTER (R1) to be used

	ROUTER(R1)
	HOST (B3) ON A DISTANT NET (N4)
	IP of the next ROUTER (R2)

	ROUTER(R3)
	HOST (B3) connected to a N/W directly connected to the Router (N4)
	


  ARP Components                

· ARP Cache: To reduce communication cost – A cache of recent IP-to-Physical address bindings is maintained.

· Technique of Soft State – for storing information that can become STALE, without warning, Protocols use Timers.

 Preset time elapses->information deleted

Advantages of Soft State

1. Autonomously determines validity; no need for a reference to an external host.

2. No dependence on n/w hardware.

Disadvantage – The sender may go on sending packets, during the validity period, even if the Dest Host has crashed. 

Cache Manager: cache control module

Cache table/ARP table
Processing an Incoming ARP message

Input Module –

· Extract the sender’s address and if the cache contains the item, update it.

· Examine operation 

 If ARP request –

   
Compare target IP address with the local IP address. 

        If identical – 

Reverse the sender & target binding.

Insert the hardware address.

Change Operation field to 2.

Send the response.

If not identical STOP.

 If ARP Response If ARP Response –

 
  The system may have been waiting for it.

 If the state is Pending

· Update the entry.

· While the Q is not empty DeQ one packet, send the packet. 

If the state is Resolved 

· update the entry.


ARP SYSTEM: 
                    OUTPUT MODULE 
                    CACHE TABLE
                    CACHE CONTROL MODULE 
                    QUEUES (One for each destination)
                     INPUT  MODULE 

CACHE TABLE:

· The fields in the Cache Table are as follows: 
  1. STATE : RESOLVED , PENDING, FREE 
  2. Queue number 
  3  Number of Attempts*
  4 Time Out. 
  5 IP Address 
  6  Hardware Address
   7. Last use: set to 0 initially and made equal to time when the row is used

8.Revalidation Timer : made equal to 50 to 75% of Timeout.

  9.Hardware Type Address 
  10.Protocol Type 
  11.HA length in bytes 
 12.PA length in bytes 
 13.Interface number (Multi-homed host Routers) 

  *Either a maximum no. of attempts or a timeout for a pending entry is provided. Usual value of this timeout is 3 minutes. 

CACHE CONTROL MODULE 


  It periodically ( say every 5 seconds), checks the cache table.  It updates the fields of the table appropriately.
Jitter  and Revalidation Timer:

· Jitter: Change in packet transfer time: 

· For some packets, transfer may be preceded by ARP request and response.

· If the hardware address is available in the Cache Table, the packet will be directly transferred.

· Timeout can become zero at any time, after which the ARP request and response may be required.

· To avoid jitter: Revalidation Timer: 

Revalidation Timer

On expiry of Revalidation Timer, CCM

· checks whether the hardware address has been used recently. 

· If so, when the RT times out, 

· a new Request is sent out ( even when no new packet for outputting is there.)

· If Timeout becomes 0, while we are waiting for a response to the request, the row is not cleared unless the max no of attempts have been tried.

Pseudo code for updating the cache table:
CCM  periodically ( say every 5 seconds), checks the cache table.  It updates the fields of the table as follows:
· If the STATE is PENDING 
                    1- Increment the No. of Attempts by one. 
                    2 If No of Attempts  > Max 
                                a- change the state to FREE 
                                b- Destroy the corresponding queue
                    3- if not send another ARP Request 

· If the state is resolved

· On receipt of a new response (to RT time out request ?) both the timers are reset.

· Decrement the value of Timeout and RT by elapsed time. ( Do not decrease below 0.)

· If RT times out, put Attempts = 1 and send an ARP request.

· If RT = 0 AND if Timeout > 0, increments the No. of Attempts by one.

· If Timeout <=0   AND if Attempts >= Max,

                                 - change the State to FREE & destroy the corresponding queue.

A router has multiple ARP modules, each with its own ARP table.

     QUEUES: 
                One for each destination 
  

      

OUTPUT MODULE: 
(Input to the Module: from IP layer)
 STEPS:      Check the cache. 
         If Found - If State is resolved extract HA and Send. 

- If state is pending enqueue the packet. 
         If not found- 
             1- Create a Q. 
             2- Enqueue the Packet. 
             3 Create a cache entry with STATE Pending, ATTEMPTS one 
             4. Send an ARP Request. 


Cache control – Sends ARP requests, if more than one Attempt is required   

[image: image1.jpg]pplication layer

ransportlayer

AYER

OUTPUT MODULE

c
< cace
n e
INPUTMODULE
7 S N
AEP AEP  ARP ARPREQ —
REQUEST RES REQ

DATA LINK LAYER

PHYSICAL INTERFACE LAYER

et
Intes
laye




Example:

Immediately after a cache control update, the cache is as follows:

State 
Q  
Att  
T-O   
IP-add      


HA

  R         2      
-    
800    
141.34.0.1       
 38:AC:45:63:AE:38

  P         5      
1      
-       
141.34.8.251       
 -

   F         -      
 -      
 -             -                   

-

   P         8       
9      
-       
141.34.56.3        
 -

   R        12     
 -      
50     
141.34.56.88    
38:AC:45:45:95:85 

State: Resolved/Pending/Free; Q: queue number; Att: No. of  Attempts; T-O: TimeOut; IP-add: IP address; HA: Hardware Address
Given that Max Time-out 20  minutes, Max number of attempts 10 
 The Cache Control Module updates the Cache Table every 10 seconds. 

STEP:(1) 8 seconds after CCU: Output Module receives an IP packet with destination address of 22.38.56.88 
STEP:(2) 25 seconds later :Input Module receives an ARP response for 141.34.8.251 with an address of E3:B5:38:A9:49:C3 
STEP:( 3) 15 second later : Output module receives an IP packet with destination address of 12.0.28.38 
STEP:(4). 26 seconds later : Input Module receives a ARP Request from 
         IP  addr;141.34.8.38 
         HA : 25:35:45:55:65:75   But the target IP address is not of this machine 
Draw the cache table after the request is received and processed. 

APPLICATION





TRANSPORT





INTERNET





NETWORK


INTERFACE





HARDWARE





ARP





FTP


Client





TCP





IP





Ethernet


DRIVER





FTP


Server





TCP





IP





Token


Ring


DRIVER





TOKEN  RING





NETWORK





Ethernet


DRIVER





TOKEN RING


DRIVER





IP





R1





A





B1





B2





R2








N3





R3





B3








HA





DEST








HA





SRC








T


YPE





H-





T


Y


P


E





P-





T


Y


P


E





H


-


S


I


ZE





P


-


S


I


ZE





O


PER


ATION











HA





SENDER








IP





ADD





SEN-


DER








HA





TARGET








IP





ADD





TAR


GET








C





R





C





P


A


D


D


I


N


G








